Agentic AI–Driven Fault Detection in Continuous Biomanufacturing (KTB1)

The system combines:

Pairwise LSTM agent ensembles,

Global multivariate feature scaling,

MQTT-based live communication between Python and Simulink, and

Real-time inference using voting across 36 binary LSTM agents,

to detect operational modes (OM0-OM8) and sudden faults in a continuous bioprocess environment.

Dataset Overview

The repository uses operational data from the KTB1 continuous biomanufacturing process.

Files

OM0.xlsx corresponds to normal operation

OM1–OM8.xlsx correspond to specific fault scenarios

Mapping from OM file → fault type is defined in ORCA Problem Statement.docx

All files contain multivariate time-series sensor data recorded from the bioreactor system, including flow, pressure, temperature, and concentration signals.

Code Overview

This code contains three major subsystems:

________________________________________

1. Python Fault-Detection Engine

Key Components

● Global Feature Scaler

Loads all OM0–OM8 datasets

Constructs a shared StandardScaler

Saves feature_names.json and scaler_mean_std.json

● Pairwise LSTM Agents

36 binary LSTM classifiers (OM_i vs OM_j)

HUMO-based balancing of minority sequences

Sequence generation with window size = 10

Each model saved under:

Agent_OMi_vs_OMj_artifacts/model_saved.keras

________________________________________

● Live Inference Engine

Subscribes to MQTT topic fault_detection/live/features

Maintains a rolling history buffer

Runs all LSTM agents → majority vote → winner class

Publishes results to:

fault_detection/live/result

2. MATLAB/Simulink Integration

MATLAB implements a real-time bridge between Simulink and the Python inference engine.

Main Components

● FaultDetectionBridge.m

- A System object used inside Simulink:

- Publishes selected Simulink signals as JSON via MQTT

- Maps Simulink vector → named features using feature_names.json

- Receives Python inference and outputs:

- FaultID (0–8)

- FaultProbability (0–1)

● getGlobalMqttClient.m

- Creates and reuses a persistent MATLAB MQTT client.

● mqtt_fault_result_callback.m

- Decodes Python results and updates MATLAB base workspace variables.

● KTB1_Fault_activation.m

- A two-phase simulation script that:

- Runs the system in normal mode until a chosen time

- Activates user-selectable faults

- Hands over state between phases

- Allows the Python LSTM ensemble to analyze both regimes

Installation

Python

python -m venv venv

source venv/bin/activate     # Windows: venv\Scripts\activate

pip install -r requirements.txt

Ensure that TensorFlow, paho-mqtt, pandas, numpy, and scikit-learn are installed.

MATLAB

Requires:

Simulink

MQTT Toolbox / mqttclient (R2022b+)

System objects enabled

Keep all files in the same file directory.

Usage

1. Train All LSTM Agents

Edit inside main():

RUN_TRAINING = True

DATA_DIR = r"__________"   %location of the file directory where all the files are saved

Then run:

python lstm_pairwise_main.py

This will:

Fit the global scaler

Train all 36 pairwise agents

Save artifacts to disk

Publish training summaries to MQTT

2. Start Live Inference

Set:

RUN_TRAINING = False

Then:

python lstm_pairwise_main.py

Python waits for live features from Simulink.

3. Run Simulink with Fault Activation

In MATLAB:

KTB1_Fault_activation

sim('KTB1_F')

Simulink will:

Publish features continuously

Receive real-time fault predictions

Output FaultID and Probability signals